ITEM MO-701   PIPE FOR STORM DRAINS AND CULVERTS

DESCRIPTION

701-1.1   This item shall consist of the construction of pipe culverts and storm drains in accordance with these specifications and in reasonably close conformity with the lines and grades shown on the plans. The pipe materials shall conform to the requirements of the 2004 Missouri Standard Specification for Highway Construction (MSSHC), Section 725 - Metal Pipe and Pipe-Arch Culverts, Section 726 - Rigid Pipe Culverts, Storm Drains and Sewers, and Section 732 - Flared End Sections.

All construction methods, testing, and acceptance criteria shall be in accordance with the standards included within this Item MO-701.

MATERIALS

701-2.1 PIPE MATERIALS.   	Pipe materials shall conform to the requirements of the 2004 MSSHC, as follows:

Corrugated Metallic-Coated Steel Pipe		Section 725

Reinforced Concrete Pipe - Class [          ]	Section 726

Flared End Sections				Section 732

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

The Engineer shall specify the class of pipe for each size specified in the plans.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

Prior to the use of materials, the contractor shall furnish manufacturer’s certified test reports to the Engineer for those materials proposed for use during construction. The certified test reports shall include a statement that the materials meet the specification requirements.

701-2.2  CONCRETE.   Concrete for pipe cradles shall have a minimum compressive strength of 2000 psi at 28 days and conform to the requirements of ASTM C 94.

CONSTRUCTION METHODS

701-3.1  EXCAVATION.   The width of the pipe trench shall be sufficient to permit satisfactory jointing of the pipe and thorough tamping of the bedding material under and around the pipe, but it shall not be less than the external diameter of the pipe plus 6 inches on each side. The trench walls shall be approximately vertical.

Where rock, hardpan, or other unyielding material is encountered, the Contractor shall remove it from below the foundation grade for a depth of at least 12 inches or one-half inch for each foot of fill over the top of the pipe (whichever is greater) but for no more than three-quarters of the nominal diameter of the pipe.  The width of the excavation shall be at least 1 foot greater than the horizontal outside diameter of the pipe. The excavation below grade shall be backfilled with selected fine compressible material, such as silty clay or loam, and lightly compacted in layers not over 6 inches in uncompacted depth to form a uniform but yielding foundation.

Where a firm foundation is not encountered at the grade established, due to soft, spongy, or other unstable soil, the unstable soil shall be removed and replaced with approved granular material for the full trench width. The Engineer shall determine the depth of removal necessary. The granular material shall be compacted to provide adequate support for the pipe.

The excavation for pipes that are placed in embankment fill shall not be made until the embankment has been completed to a height above the top of the pipe as shown on the plans.

701-3.2  BEDDING.  Bedding for reinforced concrete pipe will be classified as Class A, Class B, or Class C. When no bedding class is specified or detailed on the plans, the requirements for Class C bedding shall apply.

- Reinforced Concrete Pipe.

Class A bedding shall be used if, in the judgment of the engineer, soil conditions are such that a firm bed cannot be otherwise secured. The pipe shall be laid in the center of a concrete cradle having a minimum width of 6 inches greater than the outside diameter of the pipe. The minimum thickness of the cradle under the bottom of the pipe shall be 1/4 of the internal diameter of the pipe, and the cradle shall extend up the sides of the pipe for a height equal to 1/4 its outside diameter. The concrete shall meet the requirements of item MO-610.

Class B bedding shall consist of a bed of granular material having a thickness of at least 6 inches below the bottom of the pipe and extending up around the pipe for a depth of not less than 30 percent of the pipe’s vertical outside diameter. The layer of bedding material shall be shaped to fit the pipe for at least 10 percent of the pipe’s vertical diameter and shall have recesses shaped to receive the bell of bell and spigot pipe. The bedding material shall be sand or selected sandy soil, all of which passes a 3/8 inch  (9mm) sieve and not more than 10 percent of which passes a No. 200 (0.075 mm) sieve.

Class C bedding shall consist of a soil foundation shaped to fit the lower part of the pipe exterior for at least 10 percent of its overall height, and shall afford a uniformly firm bed throughout its entire length. In lieu of Class C bedding, Class B bedding may be used at no additional cost to the Sponsor.

- Corrugated Metal Pipe.

For corrugated metal pipe, the bed shall be roughly shaped to fit the pipe, and a bedding blanket of sand or fine granular material shall be provided as follows:

| Pipe Corrugation Depth (in.) | Minimum Bedding Depth (in.) |
| --- | --- |
| 1/2 | 1 |
| 1 | 2 |
| 2 | 3 |
| 2 1/2 | 3 ½ |

701-3.3  LAYING PIPE.  The pipe laying shall begin at the lowest point of the trench and proceed upgrade. The lower segment of the pipe shall be in contact with the bedding throughout its full length. Bell or groove ends of rigid pipes and outside circumferential laps of flexible pipes shall be placed facing upgrade.

Paved or partially lined pipe shall be placed so that the longitudinal center line of the paved segment coincides with the flow line.

Elliptical and elliptically reinforced pipes shall be placed with the manufacturer's top of pipe mark within five degrees of a vertical plane through the longitudinal axis of the pipe.

701-3.4  JOINING PIPE.   All joints shall be sealed with an approved plastic compound, cement mortar or tubular joint seal. Rubber gasketed joints may be used at no additional cost to the sponsor. Where permissible lift holes have been used, the holes shall be carefully filled with expansive mortar to provide a watertight section. The mortar shall be finished flush on the inside of the pipe and shall be properly cured on the outside. Lifting devices shall have sufficient bearing on the inside of the pipe to avoid damage resulting from a concentration of stresses around the lift holes.

If rubber gasket type pipe is specified or used, the joints shall be installed in accordance with the manufacturer’s recommendations to ensure that joint devices are properly installed and that rubber gaskets are not displaced.

In sealing rigid pipe with mortar, the mortar contact areas of all pipe ends shall be damp when mortar is applied. After applying mortar to the entire interior surface of the bell or groove, the spigot or tongue end shall be forced into position. Any remaining void in the bell or groove shall be filled with a hub of mortar built up adjacent to the bell, or a bead of mortar built up around a groove-type joint. The interior joints of either type of pipe shall be finished flush with the surface of the pipe. Outside surface of mortar joints shall be cured with membrane curing compound.

In sealing rigid pipe with plastic joint compound, trowel grade compound shall be applied to the mating surfaces of both the tongue and groove, or to the entire interior surface of the bell and the upper portion of the spigot. Rope or tape type plastic compound shall be applied in accordance with the manufacturer’s recommendations. The joints shall be forced together with excess compound extruding both inside and outside the joint. Excess compound shall be removed from the interior surface where accessible. Tubular joint seals shall be installed in a manner as recommended by the manufacturer. The joint between the bell and spigot shall be uniform for the full circumference and care shall be taken to prevent the bell from supporting the spigot.

In joining corrugated metal pipe, the ends shall be butted as closely as the corrugations will permit and shall be joined with a firmly bolted coupling band of the same material as the pipe.

701-3.5  BACKFILLING.   Pipes shall be inspected before any backfill is placed; any pipes found to be out of alignment, unduly settled, or damaged shall be removed and relaid or replaced at the Contractor's expense.

Material for backfill shall be fine, readily compatible soil, or granular material selected from the excavation or a source of the Contractor's choosing. It shall not contain frozen lumps, stones that would be retained on a 2-inch sieve, chunks of highly plastic clay, or other objectionable material. No less than 95 percent of a granular backfill material shall pass through a 1/2 inch sieve, and no less than 95 percent of it shall be retained on a No. 4 sieve.

When the top of the pipe is even with or below the top of the trench, the backfill shall be compacted in layers not exceeding 6 inches on both sides of the pipe and shall be brought up one foot above the top of the pipe or to natural ground level, whichever is greater. Care shall be exercised to thoroughly compact the backfill material under the haunches of the pipe. Material shall be brought up evenly on both sides of the pipe.

When the top of the pipe is above the top of the trench, the backfill shall be compacted in layers not exceeding 6 inches and shall be brought up evenly on both sides of the pipe to 1 foot above the top of the pipe. The width of backfill on each side of the pipe for the portion above the top of the trench shall be equal to twice the pipe's diameter of 12 feet, whichever is less.

All backfill shall be compacted to the density required under Item MO-152.

METHOD OF MEASUREMENT

701-4.1   The length of pipe shall be measured in linear feet of pipe in place, completed, and approved. It shall be measured along the centerline of the pipe from end or inside face of structure to the end or inside face of structure, whichever is applicable. The several classes, types and size shall be measured separately.

701-4.2   The number of concrete and/or corrugated metal pipe flared end sections of each class, type, and size shall be measured by the number per each installed and approved by the Engineer.

BASIS OF PAYMENT

701-5.1   Payment will be made at the contract unit price per linear foot for each kind of pipe of the type and size designated, and per each for each kind of flared end section of the type and size designated.

These prices shall fully compensate the Contractor for furnishing all materials, excavation, bedding, installation of these materials, backfilling, and for all labor, equipment, tools, and incidentals necessary to complete the item.

Payment will be made under:

Item MO-701-5.1	[        ] Corrugated Metallic-Coated Steel Pipe--per linear foot

Item MO-701-5.2	[        ] Corrugated Metallic-Coated Flared End Section--per each

Item MO-701-5.3	[        ] Reinforced Concrete Pipe--per linear foot

Item MO-701-5.4	[        ] Reinforced Concrete Pipe Flared End Section--per each

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

The Engineer shall specify the size for each pipe specified in the plans.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *