SCOPE OF SERVICES

# Project Description

MoDOT Job Number J8I3044 has been developed to scope roadway improvements on Interstate 44 from Route 360 north of Republic to Route 125 in Strafford in Greene County, Missouri.  Project limits may be extended pending funding scenarios.

MoDOT Job Number J8I3044C has been developed to construct roadway improvements on Interstate 44 from Route H (Glenstone Avenue) to Route 65 in Springfield, Missouri.  The scope of the project includes widening the existing roadway from 4 lanes to 6 lanes (with the widening on the median side).  The design needs to take into consideration future auxiliary lanes.

MoDOT Job Number J8I3225 has been developed to resurface the existing pavement on Interstate 44 from 0.6 miles west of Route 266 (Chestnut Expressway) to Route H (Glenstone Avenue) in Springfield, Missouri and from Route 65 to 0.5 miles east of Route 125 in Greene County, Missouri.

MoDOT Job Number J8S3156 has been developed to replace Bridge No. A0441 on County Road 127 (Melville Road) that crosses Interstate 44 in Greene County, Missouri.  The design needs to take into consideration pedestrian and bicyclist facilities.

The CONSULTANT shall provide the professional, technical and other personnel resources, equipment, materials and all other things necessary to prepare the bridge memo, conceptual report, preliminary roadway and bridge plans, Right of Way plans and construction roadway and bridge plans and documents, utility coordination and construction inspection.

The CONSULTANT shall perform the following services, all in accordance with the standard practice of the Commission as outlined in the MoDOT Engineering Policy Guide (EPG) and using AASHTO Green Book and other resources as listed in the EPG.

A more detailed description of the process and requirements used by MoDOT for completion of the design may be found in the EPG. The consultant is encouraged to review the appropriate sections of the manual as a means to supplement the information contained in the scope of services and provide additional guidance in the requirements and expectations of MoDOT for completion of the design services.

Services rendered by the CONSULTANT, which are considered additional services, will be addressed per paragraph (3), Additional Services of the Project Design Consultant Agreement.  The provisions of the Design Consultant Agreement outlining the responsibilities of the CONSULTANT regarding the quality and accuracy of the deliverables and products shall apply to any decisions regarding determinations of additional services.

Preparation of a supplemental agreement is necessary prior to performance of any work, which is considered as additional services, not included in the original scope of services. The consultant will not be compensated for additional services performed prior to execution of a supplemental agreement.  Only additional services, which are required due to changed or unforeseen conditions or are due to a change in the specified end product, will be considered for inclusion in a supplemental agreement.

The CONSULTANT shall prepare all plans through use of a Computer Aided Drafting (CAD) program. The CONSULTANT shall conform to the Missouri Department of Transportation Specifications for Computer Deliverable Contract Plans as referenced in the MoDOT EPG. The CONSULTANT shall use Microstation and PowerGeopak SS10.

The CONSULTANT shall review "as built" plans, aerial photographs, manuscripts, etc. and other information to be provided by the Commission and make the necessary field investigations to assure that there have been no significant changes since the information was recorded or obtained.

The CONSULTANT shall prepare a comprehensive design criteria memorandum for this entire project and submit it to the COMMISSION for review and approval. Any deviations from Commission established procedures for design, construction or materials shall be approved through the MoDOT project manager and documented by the CONSULTANT. This documentation shall include a brief justification for the deviation and the signature of the CONSULTANT project manager.

This scope of services is intended to be an accurate description of the items and tasks required for completion of the design of this project. However, each project is unique and may require more or less effort in an individual task to complete the design. The following information will explain and define in general terms the major design items of importance relating to this project. All the elements of work that are necessary to satisfactorily complete the design of this project will be listed.

# Project Management and Coordination Tasks

The CONSULTANT shall perform the following Administration/Project Management and Coordination tasks:

- Coordinate with MoDOT Project Manager to schedule and plan the kick-off core team meeting.

- Participate in the kick-off core team meeting, inviting any sub consultants that need to attend.  It is assumed that this will be a virtual style meeting.

- Schedule, plan and participate in virtual core team meetings with MoDOT.  A total of five (5) virtual core team meetings is anticipated.   A total of three (3) virtual temporary traffic control meetings is anticipated.

- Prepare minutes for all meetings and provide to core team.

- Prepare monthly progress reports that include the following:

  - Past month’s activities/accomplishments

  - Pending issues and decisions

  - Problem areas and recommended corrective actions

  - Schedule summary status (chart showing baseline schedule vs. actual schedule)

  - Next month’s planned activities/goals

  - Summary of coordination efforts (including correspondence summaries)

- All of the above shall include/address sub-consultant tasks.

- Public Involvement

The COMMISSION will be the main point of contact for receiving calls from the public.  The CONSULTANT will interact with external agencies and the public as required to accomplish the scope of services of this contract.

- The CONSULTANT will be required to attend meetings with regulatory agencies, organizations, county officials, local municipalities, property owners and other entities as required.  It is anticipated that one (1) meeting will be required for J8S3156 and one combined meeting (1) will be required for J8I3044, J8I3044C and J8I3225.

- The CONSULTANT will be required to participate in public meetings/hearings.

  - It is anticipated that one (1) design virtual public meeting/hearing will be required for J8S3156.

  - It is anticipated that one (1) combined design virtual public meeting/hearing will be required for J8I3044, J8I3044C and J8I3225.  This public meeting/hearing will require a supplemental agreement.

- The CONSULTANT shall participate in planning meetings with MoDOT personnel before meetings and public meetings/hearings.

- The CONSULTANT shall provide exhibits for the virtual public meeting/hearing based on conceptual/preliminary drawings as requested by the COMMISSION.

- The CONSULTANT shall provide the COMMISSION a database containing all property owners contiguous to the project, or within a reasonable distance of the project.  The database shall be of similar format and contain the same information as the example database provided by the COMMISSION to the Consultant. The database shall also designate whether the individual is someone the Commission will need to obtain right of way and/or easements from.

- The COMMISSION shall advertise for meetings, obtain the meeting location and room, produce copies of handouts and perform mass mailings of notices of meetings or hearings, and newsletters.

- The COMMISSION shall record and prepare the meeting minutes of the public meeting and shall prepare the transcript, if applicable.

# Environmental Services

- The conceptual stage Request for Environmental Services (RES) has been submitted for each project.  The CONSULTANT shall review the comments provided within the RES.  The CONSULTANT shall provide requested information and responses to the MoDOT Environmental Staff.

- The CONSULTANT shall submit the preliminary stage RES upon approval of the conceptual report and upon determination of proposed scope for each project.

  - The CONSULTANT shall note any anticipated right of way and/or easement acquisition needs in this submittal.

  - The CONSULTANT shall discuss the anticipated needed A-date with MoDOT’s Project Manager, and ensure the date is entered with this submittal.

- The CONSULTANT shall submit the Right of Way stage RES for each project upon approval of the preliminary plans.

- The CONSULTANT shall submit the final stage RES for each project at least 90 days prior to the PS&E deadline.

- The CONSULTANT shall review the comments provided by MoDOT’s Environmental Staff with each RES submittal. The CONSULTANT shall provide any requested information to MoDOT’s Environmental Staff, the MoDOT Project Manager and Transportation Project Designer. Items that may need to be addressed include historical buildings, bridges and sites, archeological sites, conversion of farmland, endangered species, wetlands and parklands.

- The CONSULTANT shall submit another RES when there are changes to the impacts or design from the previous submittal.

- If more than twelve (12) months lapse between required RES submittals, the CONSULTANT shall submit another RES.

- The Section 106 work will be carried out by, or under the direction of, an individual who meets the Secretary of the Interior’s Professional Qualification Standards.

- The CONSULTANT will consult with the MoDOT Historic Preservation Section (HP) to establish the area of potential effects (APE) for the archaeological and architectural surveys prior to the initiation of any work.

- MoDOT HP will prepare Tribal Notification for Federal Highway Administration to submit to interested Tribes.

- The CONSULTANT shall then develop a concise historic context to guide the investigations to identify and evaluate historic properties (or explain their absence) in the APE and submit a written Research Design to MoDOT HP, staff before conducting archaeological and architectural surveys.

- The CONSULTANT will conduct an architectural survey that follow the MoDOT Built Environment Resources Methods and will identify and document all architectural resources (i.e., buildings, structures, objects, sites, and districts/landscapes) and all bridge resources (i.e., highway, railroad and pedestrian bridges, viaducts and culverts, excluding metal, plastic and reinforced concrete pipes) that are forty or more years of age located within the APE. The CONSULTANT shall contact MoDOT HP regarding National Register of Historic Places (NRHP) eligibility recommendations for all bridges.

- The CONSULTANT will complete the archaeological investigation within the APE to identify prehistoric and historical sites following MO SHPO’s Guidelines for Phase I Archaeological Surveys and Reports.  For archaeology, this current scope of services covers only consultant services through the Phase I survey and the subsequent Phase I survey report. This can include preliminary eligibility assessments or recommendations for additional (i.e. Phase II) work. Sites form, for new and revisited sites, shall be completed; after consulting with MoDOT HP, the forms can be submitted directly to MO State Historic Preservation Office (SHPO).

- The results of the built environment (e.g., buildings, bridges, etc.) and archaeological investigations including effects on resources eligible for listing on the NRHP,  should be presented by the CONSULTANT in a single draft report to MoDOT HP, unless the reasons for separate documents has already been approved by MoDOT HP.  Once the report is finalized, MoDOT HP will submit it to SHPO. Recovered artifacts, archaeological field notes and lab forms, either originals or copies, shall be submitted to MoDOT HP upon acceptance of the final report.

- MODOT will coordinate with the SHPO regarding potential cultural, historical, and archeological issues.

# Survey

- MoDOT will provide the available survey data for each project.

- MoDOT will provide any additional survey work needed for the design of each project.

  - The CONSULTANT shall submit a Survey Request to MoDOT’s Transportation Project Manager that includes detailed information on the survey work needed.

  - The CONSULTANT shall provide an exhibit (pdf, map or kmz file) that shows the outline and/or location of the requested survey.

# Utility Coordination

The CONSULTANT shall lead the effort on utility coordination. Phase 1 efforts will identify utility type for the conceptual report. Phase 2 assignments will include working with the professional land surveyors for the following project documentation and plan details.  A supplemental agreement will be required for Phase 2 activities.

- Review completed topographic surveys to develop preliminary plans, right-of-way plans and final roadway plans, including any necessary improvements and existing topography within the limits of the project. Topographic surveys shall consist of pertinent topographic features, but not limited to:

- Existing drainage and sanitary structures (pipes, types, flowlines, sizes) where pipe improvements were identified during the concept and preliminary phases

- All identifiable utility locations and equipment at signalized intersections and longitudinal areas in between intersections that are adversely impacted

- Existing signal, lighting and any ITS equipment inside slope limits

- Sign posts, size, identification and photo log

- Field locate visible above ground evidence of utilities located within the project area. “Missouri One Call” and MoDOT will be contacted and a formal request will be submitted for marking the locations of member utilities. In the event that “Missouri One Call” fails to respond, in whole or in part, to the formal request, underground facilities, structures, and utilities will be plotted from surveys and/or available records. The locations of all utilities are to be considered approximate. There may be other utilities, whose existence may not be known at the time of the survey.

- Consult with MoDOT’s Utility Engineer to coordinate with surveyors on underground utility locates.

- Coordinate to obtain One-Call tickets to have utilities located in identified areas of proposed project.  An estimate of 1 MO One Call ticket for every 400 linear feet of sidewalk is reasonable for preliminary design. Additional MO One Call tickets after preliminary design is complete will be required where additional items were added to the final design plans compared to the concept design plans.

- Coordinate with surveyor to complete utilities survey and verify completeness and accuracy of utility topographical survey.  Service connections shall be included as part of the utility survey.

- Coordinate with utility companies on the development of the plan of adjustment and obtain cost estimates for reimbursable utilities for the District Utility Engineer’s approval.

- Show the existing utility facilities and plan of adjustments for proposed utilities facilities in the contract plans (plans sheets, cross sections, culvert sections).

- Work with the various utility owners to determine if any needs for renewals within the project limits.

- Coordinate relocation of each impacted utility on the project during design and construction.

- Prepare special utility sheets as necessary (including utility profile and exhibits).

- Assist District Utility Engineer and/or Coordinator in the preparation of agreements.

- Identify locations for power service needs, prepare service request for submittal and coordinate with the power company to obtain estimated costs.

- Coordinate with MoDOT District Utility Engineer regarding the need for test holes and initiating the potholing with the applicable utility owner.  Provide test hole information at critical utility locations to the District Utility Engineer.

- Prepare utility job special provision and information for the preparation of the Utility Status Letter for District Utility Engineer.

- Provide assistance and answer utility related questions during the construction phase for MoDOT staff and the roadway contractor.

- Review MoDOT excavation permits in project area for conflicts with design and construction phase.  This includes new utility installation, modifications to existing facilities, and any developer driven improvement within the project limits.

# Geotechnical Services

- The CONSULTANT will provide equipment and staff to drill soundings for bridge replacement, MSE walls and sound walls, as required and provide a final foundation investigation report for J8S3156, J8I3044 and J8I3044C.  The final report will be delivered on DATE TBD.

- The replacement of the existing bridge for J8S3156 will be determined as part of this contract.  It will be on the existing bridge alignment.  The proposed bridge will account for future I-44 widening and auxiliary lanes.  The location and design of the MSE walls will be determined through this contract.  The CONSULTANT will be provided the Noise Study for inclusion of any required sound walls.  The CONSULTANT shall provide preliminary geotechnical information in accordance with EPG Section 320.1, the requested properties on Form A of the Bridge Division Request for Soil Properties in accordance with EPG Section 320.2, and seismic properties as requested on Form B.

- The CONSULTANT shall assess stability of the proposed fill as well as the potential for settlement in accordance with EPG Section 321.1.  The CONSULTANT will provide the total settlement, duration of settlement, any need for ground improvements or other special construction considerations during construction of the proposed embankments.

- The CONSULTANT will perform standard penetration testing (SPT) and split-barrel sampling in accordance with ASTM D1586 using an automatic hammer in accordance with section 7.4.1 Method A.  The automatic hammers shall be calibrated in accordance with manufacturer’s recommendations and ASTM D4633 at least every 2 years or sooner as required therein.  The calibration report shall be prepared in accordance with ASTM D4633 and shall be signed and stamped by a professional engineer.  During field exploration, the automatic hammer shall be run in accordance with manufacturer’s recommendations and in accordance with the settings and rates that were used during the hammer calibration.

- The CONSULTANT shall provide, at a minimum, a geologist registrant in training (GRIT) or an engineer in training (FE) to log the borings in the field per MoDOT’s logging protocol.  The engineer or geologist shall have at least 2 years of experience logging boreholes.  Logs shall be reported in gINT format.  MoDOT will provide preferred gINT templates when requested.  At final submittal, please provide a copy of the electronic gINT file, in addition to the final report deliverables.

- The CONSULTANT shall provide the following information on their boring logs:

- N value of blows per foot

- N60 value of blows per foot (corrected for the energy efficiency of the auto- hammer)

- Energy efficiency of the auto hammer

- Drilling equipment identification

- Boring locations (Stations and/or Coordinates, and Elevations with datums)

- The C/L of the proposed alignment and bridge boring locations on Form A will be established by MoDOT prior to drilling, although it may be necessary for the CONSULTANT to offset boring locations in the field and determine new offsets and elevations.  MoDOT will not provide information for the new offsets and elevations.  It will be necessary to coordinate surveying with Lydia Brownell, a minimum of three weeks before locations are required.

- Traffic Control will be provided by the CONSULTANT.  The CONSULTANT will be responsible for attaining all necessary permits to perform the work. Public utilities shall be notified via Missouri One-Call before drilling begins.  The CONSULTANT will be responsible for backfilling holes and patching the bridge deck if necessary.

### Laboratory Testing

- The CONSULTANT shall provide all testing necessary to provide the requested properties.  All lab test data shall be incorporated in the finished log of borings or a copy of the data provided if not feasible to include within the boring log.  All testing shall be completed by an AMRL certified laboratory.

- Fertility samples will be collected by the CONSULTANT and sent to MoDOT’s Central Laboratory for testing.  The COMMISSION will provide the seeding report based on the fertility samples collected.

### Items Provided by the Commission

- Existing bridge plans

- KMZ file – Bridge, MSE Wall and Sound Wall Locations

- Sounding Layout and Request for Soil Properties for A9147

- Bridge aerial layout

- Sounding locations will be staked by the COMMISSION and a copy of field survey information

All correspondence and communications between the COMMISSION and CONSULTANT regarding this Memorandum of Understanding shall be directed through the following contacts:

COMMISSION:	Lydia Brownell, P.E.

Lydia.Brownell@modot.mo.gov

Missouri Department of Transportation

Geotechnical Director

Construction – Materials Laboratory

1617 Missouri Boulevard

P.O. Box 270

Jefferson City, Missouri 65102

Telephone: (573)526-4628

CONSULTANT:	TBD

# J8I3044

The CONSULTANT’s attention is directed to Section 128 of MoDOT’s Engineering Policy Guide (EPG) for general guidelines and requirements for the conceptual study.

- The CONSULTANT shall review existing traffic counts, crash data, utility, drainage, right-of-way and pertinent planning information.

- The CONSULTANT shall review any recent traffic studies performed along Interstate 44 within or adjacent to the project limits.  MoDOT shall provide available traffic studies, including, but not limited to, the 2018 Springfield Freeway Study Report and the I-44/Route 13 Study that is currently in progress.

- MoDOT will provide the available Traffic Forecast Report.  Build-Year is 2028, and Design-Year is 2048.

- The CONSULTANT shall perform a detailed safety and operational analysis of the following conceptual improvements.

- Increase driving lanes from four (4) lanes to six (6) lanes by widening in the median at the following locations:

    - Route 360 to Route 13

    - Route 13 to Route H

    - Route 65 to Mulroy Road

    - Mulroy Road to Route 125

- Construct auxiliary lanes in each direction at the following locations:

    - Route 360 to Route 13

    - Route 13 to Route H

    - Route 65 to Mulroy Road

- Rebuild the existing pavement at the following locations:

    - Route 360 to Route 13

    - Route 13 to Route H

    - Route H to Route 65

    - Route 65 to Route 125

- Cost Estimate – MoDOT will provide available conceptual stage cost estimates.

  - The CONSULTANT shall evaluate and upgrade the cost estimates as needed.

  - The CONSULTANT shall provide cost estimates for any remaining conceptual improvements listed in Task 4.

  - The CONSULTANT shall provide revised cost estimates that replace gabion baskets with MSE walls.

  - A cost estimate for a pedestrian underpass between Route 13 and Route H shall be developed by the CONSULTANT.

  - A cost estimate for any recommended sound walls shall be developed by the CONSULTANT.

- Operational Analysis:  Following the Highway Safety Manual (HSM) methodologies, the CONSULTANT shall calculate the LOS and delay for each conceptual improvement listed in Task 4 for J8I3044 for the following scenarios.

  - Existing

  - Build-Year No-Build (2028)

  - Design-Year No-Build (2048)

  - Build-Year Build (2028)

  - Design-Year Build (2048)

- Safety Analysis:  Following the HSM methodologies, the CONSULTANT shall perform a predictive safety evaluation, using Crash Modification Factors (CMF’s) where available.  The CONSULTANT shall calculate the Benefit-Cost Ratio, identify safety or operational issues, and provide actual and expected crash information for each conceptual improvement listed in Task 4 for J8I3044 for the following scenarios.

  - Existing

  - Build-Year No-Build (2028)

  - Design-Year No Build (2048)

  - Build-Year Build (2028)

  - Design-Year Build (2048)

- The CONSULTANT shall prepare a 4R Conceptual Study Report, with the safety and operational analysis attached as an appendix.

- If J8I3044 is programmed as a project in MoDOT’s STIP, a supplemental agreement will be required for the development of the Traffic, Safety and Operations Report, the Access Justification Report, geotechnical services, preliminary roadway and bridge plans, bridge memos, Right of Way plans, and PS&E roadway plans, bridge plans and documents.

# J8I3044C, J8I3225 and J8S3156

The CONSULTANT’s attention is directed to Section 128, 235, 236, 237 and Category 700 of MoDOT’s Engineering Policy Guide (EPG) for general guidelines and requirements for conceptual design, preliminary plans, right of way plans, contract plans and bridge design.

- The CONSULTANT shall review “as built” plans, aerial photographs, manuscripts, etc. and other information to be provided by the COMMISSION before visiting each project site location. The CONSULANT shall perform a field investigation to assure that there have been no significant changes since the information was recorded or obtained.  The CONSULTANT shall incorporate any significant changes found during the field investigation as part of this contract The CONSULTANT shall document the extent of improvements, location of improvements (side of road, intersection/interchange quadrant, etc.), project exceptions and overall intent of the improvements.

### Conceptual Design (J8I3225 and J8S3156):

- An approved Conceptual Report for J8I3044C will be provided to the Consultant.

- The CONSULTANT shall summarize the determinations made during the scoping process in a Conceptual Study Report for J8I3225 and J8S3156.

  - The CONSULTANT shall evaluate right of way impacts of the proposed improvements and the need for additional right of way, permanent easements, and temporary easements.

  - The CONSULTANT will evaluate the location of the proposed improvements for existing utility conflicts and will document the need for relocation and coordinate with the MoDOT District Utility Engineer to develop a conceptual cost for relocation to be included in the conceptual cost estimate.

- The CONSULTANT shall prepare a separate opinion of cost for J8I3225 and J8S3156.  Available conceptual stage cost estimates will be provided by MoDOT for review and revision.

  - Estimates shall include construction costs, right of way costs, and the potential for any utility adjustments.

  - The CONSULTANT shall prepare using BidTabPro format, utilizing district, county and statewide averages for item costs.

  - If needed, square foot right of way cost estimates and utility relocation costs shall be provided by MoDOT.

- The CONSULTANT shall develop conceptual drawings to be included in the Conceptual Study Report, and to the quality and detail that they can be included in any public involvement efforts.

### Preliminary Roadway Design:

  - The COMMISSION shall furnish the Consultant the pavement design recommendation and general special provisions to be used in the preliminary design for each project.

  - As applicable, the CONSULTANT shall use the Southwest District ADA Streamlining processes (Typical Sections and Temporary Traffic Control).

  - The CONSULTANT shall participate in Temporary Traffic Control meetings with MoDOT staff to determine, design and review the Traffic Control Plan for this project.  It is anticipated that there will be three (3) virtual meetings for Temporary Traffic Control.

  - The CONSULTANT shall review the Noise Study Report for J8I3044.  Preliminary design of recommended sound walls will be included in the preliminary plan submittal.

  - The CONSULTANT shall prepare the preliminary plan construction cost estimate for each project.

    - The COMMISSION will prepare the right of way cost estimate based on the right of way requirements furnished by the CONSULTANT, if needed.

    - The CONSULTANT shall prepare the cost estimate for any utility impacts. The CONSULTANT shall include utility impacts in the plans and construction cost estimate for those utility impacts that are determined to be included in the construction contract.

    - The CONSULTANT shall provide detailed estimates for each anticipated bid item.  Lump sum quantities shall not be used for the preliminary plan estimates.

  - At a minimum, the following shall be submitted for the Preliminary Plans for each project:

- Preliminary Plan Title Sheet

- Typical Sections

- Plan Sheets at 1” = 20’ horizontal (or different scale as determined by MoDOT Project Manager for clarity) and extend to 100 feet beyond the project limits.

- Special Sheets for geometrics and special design features.

- Temporary Traffic Control Plans.  The temporary traffic control plan should be summarized in the Preliminary Plans Letter of Transmittal.

- Preliminary drainage sheets (or information provided in plan sheets)

- Preliminary signing sheets

- Tentative Easement and Right of Way limits with property lines and ownership information, if applicable.

- Preliminary Cross Section Sheets at 100’ intervals, 1”=10’ (1:100), horizontal and vertical, including entrance sections with existing and proposed grades

  - The CONSULTANT shall prepare the letter of transmittal for the preliminary plans.  The COMMISSION will provide the template upon request.

### Preliminary Bridge Design

- Perform the geometric analysis at the proposed bridge site necessary to develop type, size and location drawings consisting of a general plan and elevation plan of the structures, typical roadway sections and roadway profiles.  This includes preparation of the Bridge Memorandum & Layout (including the itemized preliminary bridge estimate).

- The structure and/or box culvert type and size (if applicable) shall be based on roadway alignments, geometric analysis, hydraulic analysis, spill slope requirements, roadway grades and/or clear zone requirements.

- The superstructure type shall be dependent upon site constraints and a detailed cost analysis comparison.

- All requirements outlined in the MoDOT Engineering Policy Guide (EPG) shall be met.  The CONSULTANT shall follow MoDOT’s “practical design” philosophy and submit any design exceptions as necessary.

- The CONSULTANT shall provide one (1) electronic copy of the preliminary plans, construction cost estimate, preliminary plan checklist and letter of transmittal to the Commission electronically for review and approval, as scheduled below.

### Right of Way Plans

- The COMMISSION will provide title insurance information for parcels where new right of way is being acquired, prepare right of way appraisals and secure the necessary right of way by negotiation or condemnation, if necessary, for construction of this project.  Any ownership deeds, easements deeds, subdivision plats, etc. needed or used in the development of existing property lines for the right of way plans will be furnished by the CONSULTANT.

- The CONSULTANT shall be responsible for staking and re-staking tentative right of way on individual properties, as required by MoDOT staff, during the right of way negotiation and acquisition phase of the project.

- The CONSULTANT shall prepare right of way plans, which may be separate drawings from those used for design and construction details. The right of way plans shall show alignment, geometric design, removal of improvements, drainage facilities, property lines and ownership, ¼ ¼ section lines, subdivision lot lines, other land survey information, street lines and existing right of way and easements.

- The CONSULTANT shall include any plan details, which will require additional right of way or easements during the construction phase of the project such as bypasses, temporary erosion control, etc.  Areas of new right of way, permanent easements and/or temporary easements required from each individual property owner must be denoted underneath the Parcel Number and Name on each individual tract.

- Right of way plans shall be submitted to the COMMISSION for review and approval as scheduled in Exhibit IV. The right of way plans shall be at the same scale as the construction plans. The right of way plans shall include any design details that control the width of right of way and necessary easements.

- The following minimum design features shall be included on the right of way plans:

  - Title sheet with appropriate project limits, access note and traffic data completed.

  - Typical Sections

  - Plan and Profile Sheets – CONSULTANT shall finalize and detail the horizontal and vertical improvements.  CONSULTANT shall clearly describe important design notes and features on all plan sheets in standard MoDOT format.

    - New right of way lines and all easements shall be dimensioned from the centerline, or crossroad centerlines, if necessary.  Bearings and distances on the right of way lines will be required on the final location survey.

    - Construction limits (slope lines); drainage facilities; entrances and their reference location  along with existing and proposed entrance grades, width and type; property owners, with areas of new right of way, easements and remaining property size; centerline bearing, ties to legal land corners from centerline stations with notation for corner witness by a registered land surveyor; existing utility locations and easements, including replacement utility easements; horizontal curvature information; and proper right of way symbolization for new right of way (access control) and easements, including areas which may be required to accommodate temporary erosion control.

    - Section, Township, Range, and/or U.S. Survey information on each plan sheet near the title block or appropriate survey/section line.  The section line information will be provided to the ¼ ¼ section lines.

  - Driveway Profiles – CONSULTANT shall develop and layout horizontal and vertical alignments for all driveways within the project limits.

  - Special Sheets

  - Culvert Sections/Storm Drainage – CONSULTANT shall finalize size and length of any culverts and place description on plan sheets.

  - Cross sections – CONSULTANT shall develop cross sections for the purpose of determining right of way impacts/takes and quantities.

- If needed, the CONSULTANT shall schedule a meeting and/or field check with the COMMISSION to review right of way plans prior to the completion of the right of way plans.  The CONSULTANT shall make any necessary revisions to the right of way plans as determined by this meeting and field check.

- The CONSULTANT shall provide an updated construction cost estimate for the Right of Way design stage.

- The CONSULTANT shall submit the Right of Way plans and construction cost estimate with a letter of transmittal.  The letter of transmittal shall be provided as a word document.

- The CONSULTANT shall submit the Right of Way Plan checklist.  The COMMISSION shall provide the checklist to the Consultant upon request.

- The CONSULTANT shall be responsible for making all revisions to the right of way and construction plans due to negotiations with the property owners in an effort to acquire right of way.

- The CONSULTANT shall provide one (1) electronic set of fully signed and sealed right of way plans for the Commission’s use.

- The CONSULTANT shall provide one (1) electronic set of approved Right of Way plans and cross sections to be used by MoDOT’s Utilities Department.  The known utilities shall be color-coded on the plans.

- The COMMISSION shall review, approve and certify the right of way plans as completed by the CONSULTANT.

- The COMMISSION will prepare right of way appraisals and secure the necessary right of way by negotiation or condemnation, if necessary, for construction of this project.

### Final Bridge Design

Furnish to the COMMISSION fully checked design plans, job special provisions, design computations, quantity computations, final cost estimate, a construction work day study for the structure(s).  The CONSULTANT is expected to make the COMMISSION aware of more economical design alternatives that may become apparent during the preparation of the final design.

- The plans shall be complete and shall cover all parts of the structure they represent.  The degree of detail shall be comparable to that furnished on typical plans prepared by the COMMISSION.  High resolution final signed and sealed plans, will be submitted in Adobe Acrobat Reader format version 7 or higher.  Final signed and sealed plans shall be in pdf full size (34” x 22”) format.  These deliverables shall use the file naming convention and be in accordance with the “Specifications of Computer Deliverable Contract Plans” requirement outlined in the Commission’s Engineering Policy Guide, Section 237.13.3.  The electronic plans in Microstation format cannot be signed and sealed.  The electronic submittals shall be made in a method suitable to MoDOT.

- All construction changes made to the plans during construction of the project shall also be submitted electronically in Adobe Acrobat and Microstation format.

- The job special provisions shall be complete and describe all design features, construction procedures, or material requirements in the plans that are deviations from the latest edition of the Missouri Standard Plans for Highway Construction.  Typical job special provisions that have been developed by MoDOT for previous jobs are posted on MoDOT’s website and are available for use and modification as needed.  The job special provisions shall include a table of contents sheet that is signed and sealed by a professional engineer registered in Missouri.  The signed and sealed job special provisions shall also be submitted in Adobe Acrobat Reader format, version 7 or higher.  Job Special Provisions shall also be submitted in Microstation Word format.  The submittal letter shall explain the need for each provision.

- The design computations and plans shall be acceptable to, and will become the property of the Commission.  The CONSULTANT shall submit design computations in Adobe Acrobat Reader version 7.0 format or greater. The files shall be transferred in a manner acceptable to MoDOT.  The design computations shall contain an index file, with electronic links to the files contained within.  Submittals shall include a minimum of set of design computations for each project.  The design computations shall not be combined with the Microstation or the Adobe Acrobat Reader submittals.

- The final estimate submitted by the CONSULTANT shall include backup material that supports the estimates made for non-standard or lump sum pay items.

- The CONSULTANT shall submit the hours and cost summarizing the design effort for each bridge.  The summary shall include separate amounts for: Number of Hours for Bridge Preliminary Design, Cost of Bridge Preliminary Design, Number of Hours for Bridge Final Design, Cost of Bridge Final Design.  Generally, the above amounts should include all hours and costs invoiced that are attributable to bridge design and plans preparation up to the point of turning in the signed and sealed plans.  It should not include hours attributable to preparing the bridge survey, final construction cost estimate, or workday study.

### Final Roadway Design

- The COMMISSION will secure execution of agreements with the cities and/or county agreements. A copy of the executed agreements will be furnished to the CONSULTANT for information purposes, upon request. The CONSULTANT shall conform to all design provisions of these agreements.

- If determined necessary, a field check shall be held with CONSULTANT and MoDOT staff prior to completing final design plan quantities. The CONSULTANT shall make any necessary revisions to the final plans as determined by this design field check.

- The CONSULTANT shall utilize MoDOT’s Southwest District ADA Streamlining typical sections and methods of design and quantity calculations, as applicable.

- The CONSULTANT shall prepare detailed temporary erosion control plans for review and approval before inclusion in the final design plans.

- The CONSULTANT shall prepare computations for all design plan quantities.  All plan quantities shall be shown on the Quantity Sheets, and by construction stage, if applicable.  The format for these sheets shall be furnished by the COMMISSION.  Specialty items may have separate sheets for quantity tabulations.

- The COMMISSION shall prepare all General Job Special Provisions, which are to supersede the Missouri Standard Specification for Highway Construction.  A brief reason for the deviation from the standard plans and specifications should also be provided.  The CONSULTANT shall prepare only Job Special Provisions (JSPs) related to design elements shown in the plans.  The COMMISSION shall provide any JSPs that are required as a result of right of way acquisition, utility impacts and environmental issues.

- The CONSULTANT shall prepare for review and approval any design exceptions required for construction of the project.

- At a minimum, the final design plans shall include:

  - Title Sheet

  - Typical Sections

  - Quantity Sheets – the latest version of the temporary traffic control summary sheet

  - Plan Sheets at 1”=20’ horizontal (or different scale as determined by MoDOT Project Manager for clarity).  Plan sheets shall include all necessary adjustments to signing and proposed pavement marking.

  - Profile Sheets at 1”=20’ horizontal and 1”=10’ vertical

  - Reference Points

  - Coordinate Points

  - Culvert Sections at 1”=10’, if needed

  - Special Sheets for geometrics

  - Temporary Traffic Control Plan Sheets

  - Erosion Control Sheets

  - Lighting Plan Sheets, if needed

  - Signal Plan Sheets, if needed

  - Signing Plan Sheets, if needed

  - Pavement Marking Plan Sheets

  - Earthwork Quantities, Cross Sections at 100’ intervals, 1”=10’ (1:100), horizontal and vertical, including entrance sections with existing and proposed grades

  - Tabulation of Quantity Sheets

  - Job Special Provisions in electronic format readable in COMMISSION’s current word processor

  - File with the bid items and quantities as generated by COMMISSION’s Estimate Program

  - Equipment and Materials List D-15 – generated from the COMMISSION’s Estimate Program, if applicable

  - Construction Workday Study

  - Workzone Impact Analysis Spreadsheet

  - Traffic Management Plan

  - Electronic Deliverables as per the EPG

- Additional plans and information may be required to complete the Final Design Plans.  With the submittal of the Final Design the CONSULTANT shall also provide the COMMISSION a statement that an internal quality control check has been conducted and to the best of the CONSULTANT's knowledge the final design plans are free of gross errors, misleading or confusing typos, and includes adequate information to construct the project.

### Construction Inspection

- Purpose

  - The Consultant shall provide construction inspection, materials testing, and other services as needed.  Support services will be assigned by the MoDOT Resident Engineer, Brad Gripka, as described within this Exhibit for the Design-bid-Build projects designated as Job No. J8I3044C, J8I3225 and J8S3156 (PROJECT).

  - The Consultant shall provide support services as requested throughout the following phases of the Project:  CONSTRUCTION.

  - The Consultant shall provide PROJECT related services as described herein.

    - The consultant shall provide to MoDOT qualified inspection services to supplement MoDOT’s Resident Engineer (RE) and staff on the inspection of the above referenced project(s).

    - The services provided shall include Quality Assurance (QA) verifications on all items of installed work and QA tests as outlined in the attached Contractor Quality Control Job Special Provision (JSP).  Testing frequencies shall be no less than as defined in the Inspection Testing Plan (ITP) and in accordance with the applicable Missouri Standard Specifications for Highway Construction.  Above referenced documents can be found at https://www.modot.org.

    - Any testing performed by the consultant can only be performed by personnel certified for the tests being conducted through MoDOT’s technician certification program AND testers must carry current credentials validating their certification

    - The consultant’s inspector shall be present on the jobsite daily to capture necessary diary information for progress monitoring.  For periods of work between critical installations and at certain hold points, the inspection time spent on the project (while daily) can be minimal if approved by the RE.

    - Daily, the inspector shall provide/perform:

      - A daily work report entered into AASHTOWare Project documenting all construction activities on the project.

      - Any QA tests required by the ITP for the work being performed.

      - All test results shall be recorded in AASHTOWare Project in the proper format as defined by the Sampling Checklist and Materials Summary.

      - A work zone traffic control review of all TCDs associated to the project to ensure compliance with the plans and MUTCD.  All reviews will be maintained and available for review upon request by the RE.

    - On a semi-monthly basis, the inspector may be required to run an AASHTOWare Project estimate and submit to the RE for review.

    - As needed, develop change orders to document overruns/underruns on the project to ensure prompt payment to the contractor.

    - Expected allocation of resources are as follows:

      - Staffing of above referenced projects from March 2020 to November 2021.  Services may be extended by an amendment to the contract amount.

      - The consultant should expect to dedicate 2 individuals for a total of 40 hours each per week for the duration of the project(s) except for intervals of time that the weather will not allow the work to continue or the contractor chooses not to staff the project in question.

- Quality Assurance Personnel Duties and Responsibilities

  - Quality Assurance Testers – Quality Assurance Testers will perform testing and sampling during construction as outlined in the Inspection & Test Plan (ITP). The QA sampling and testing will be performed by QA Testers certified through the MoDOT Technician Certification Program or a certification program preapproved by MoDOT.  The QA Testers report directly to the Quality Assurance Inspector (QAI).  The QA tester can be the QAI.

  - QA Tester responsibilities:

    - Perform required tests and document test in accordance with contract documents.

    - Conduct material sampling and testing in conformance with the appropriate methods and frequencies.

    - Ensure all measuring and test equipment is properly maintained and calibrated.

    - Promptly communicate nonconforming material to RE and production staff.

  - Quality Assurance Inspectors (QAI) – Quality Assurance Inspectors will have a thorough understanding of the contract requirements and will perform inspections during construction to ensure the quality of the work meets or exceeds contract specifications. The QA Inspector will be knowledgeable of the work he/she is inspecting and will be able to recognize conforming and nonconforming work and material. The QA Inspector reports directly to the MoDOT’s Resident Engineer.

  - QA Inspector responsibilities:

    - Perform required inspections and complete checklists in accordance with contract documents and found at the following link -http://www.modot.org/business/contractor_resources/Quality_Management/

    - Ensure inspection observations are included on Daily Inspection Report (DWR).

      - Ensure material is conforming to the contract requirements before being incorporated into the work.

      - Promptly communicate nonconformance material and work to RE.

      - Wage rate inspections are required at a maximum of 1 every week per job.  QAI needs to discuss required frequency on each job with the RE prior to work to ensure accuracy.

      - CUF (commercially useful function) reviews on all DBE Subs, minimum 1/sub/job.  These must be submitted to CPOA (Construction Project Office Assistant) within 24 hours of completion.

  - Pre-Activity Meeting

    - Pre-Activity Meetings are necessary to discuss details of the Work Plan and schedule. Pre-Activity Meetings will be held 24-hours prior to the start of each new project activity or change in work crew. The Superintendent/Job Foreman of the activity will lead the meeting and others present at the meeting will be the QA Inspection and Testing Staff, and MoDOT.

  - Inspection and Testing

    - Inspection and testing are performed during all phases of the work from start to completion to ensure the work and testable material (asphalt, concrete, aggregate, etc.) meets or exceeds the contract requirements. Consultant will perform inspection and testing of the work and material as specified in the Inspection and Testing Plan and MoDOT-provided checklists. All results will be documented in AASHTOWare Project.

- Inspection and Testing Plan (ITP)

  - The Inspection and Testing Plan (ITP) outlines the acceptance criteria for contract items for this project along with the required tests, testing frequency, and the required QA documentation. The ITP is provided by MoDOT.  Any changes to the specifications, testing procedures, or the testing frequencies from the standard ITP will be implemented via change order.  In addition, a change order will be completed to add acceptance criteria for nonstandard contract items not included in the standard ITP.

  - See link for ITP: http://www.modot.org/business/contractor_resources/Quality_Management/

- Daily Work Report (DWR)

  - A Daily Inspection Report (DWR) will be completed on the project each day to document pertinent project activities. The report will include a detailed diary that describes the work performed as well as observations made by QA Inspection staff regarding quality assurance. The DWR will include other items such as weather conditions, location of work, installed quantities, tests performed, and a list of all subcontractors that performed work on that date. The DWR is completed in AASHTOWare Project.

- Hold Points

  - Hold Points are events in the work process that require approval from MoDOT prior to continuing work. Hold Points occur between different definable stages of work when the succeeding work depends on the acceptance of the preceding work. A list of minimum Hold Points is available on MoDOT’s website and can be provided upon request.  Additional Hold Points can be added by MoDOT.

  - Hold Point inspections will be at times planned in the Weekly Schedule. The Hold Points may be rescheduled as needed, but the Consultant Inspector will provide a minimum 24-hour advance notification to MoDOT, unless otherwise approved by MoDOT.

  - Prior to all Hold Point inspections, the consultant Inspection Staff will provide MoDOT with the Daily Inspection Reports, Inspection Checklists, Test Reports, and Materials Receiving Inspection Reports for the work performed leading up to the Hold Point.  MoDOT-identified issues will be corrected prior to continuing work and a new Hold Point scheduled.

- Material Receiving

  - Material Receiving is assurance that products and materials are identified from receipt and during all stages of production, delivery and installation.  At receipt, the Consultant Inspection Staff will inspect general condition of material and determine if material is compliant based on the requirements specified in the ITP, Specifications, Job Special Provisions, Standard Drawings, or other pertinent contract documents. For products that require on-site inspection by MoDOT Materials staff, the RE will notify MoDOT staff promptly to schedule the inspection.

  - Some materials are pre-qualified or pre-accepted for use on MoDOT projects.  A list of these items is maintained by MoDOT and can be found on the MoDOT internet site.  These lists include: Pre-Acceptance List (PAL), Approved Product List (APL), Bridge Pre-qualified Products List (BPPL), and the Qualified List (QL).  Although pre-qualified or pre-accepted, appropriate documentation detailed in the Specifications will accompany these products for acceptance.

  - All material delivered to the project, excluding testable material, will be inspected for appropriate dimensions, quantity, condition, markings, etc., and accompanied with appropriate documentation. All PAL items will be inspected to ensure material confirms to plans and specifications and disposition remarks added to the associated identification number in AASHTOWare Project.

- Quality Documentation

  - Project documentation (inspection checklists, daily work reports, test records, Materials Receiving Inspection Reports, etc.) referenced in this plan will be electronically stored and organized by the Consultants Inspection Staff in AASHTOWare Project.

  - Project documentation will be electronically saved to AASHTOWare Project within 48-hours after the work shift ends.

  - In addition, to the above, all project correspondence must be delivered to MoDOT electronically so it can be archived with the project’s permanent records. Copies should be maintained by the consultant for access as needed.  This includes, but is not limited to: delivery tickets, mix designs, mill test reports, certifications, test reports, contractor schedule, any letters associated to the contract, wage rate inspections, NCR’s, and order records. These items shall be uploaded and stored in eProjects.

  - Certified payrolls for contractor activity will be checked and maintained by MoDOT.

- Control of Non-Conforming Work And Material

  - Non-conformance reporting will be used to identify and ensure that work, material, and/or processes (i.e. QC inspections, tests, documentation, etc.) that fail to satisfy the contract requirements are identified and promptly resolved.  QA staff or production staff will identify non-conformances and the QAI will document using a Non-Conformance Report (NCR).  MoDOT may also initiate an NCR.

  - The NCR process is as follows:

    - Non-conforming work and/or material are identified.

    - NCR is issued on the provided standard form.

    - QAI forwards copy of NCR to MoDOT with proposed resolution.

    - MoDOT either approves or rejects the proposed resolution.  A dialogue will ensue following any rejected proposals until an acceptable solution is identified.

    - The MoDOT-approved resolution is executed.

    - New inspection and/or test of previously identified non-conforming item is completed.

    - QAI verifies previously identified non-conforming item is now compliant and requests final approval from MoDOT.

    - MoDOT evaluates previously identified non-conforming item.  If accepted, MoDOT signs NCR for closure.

# Drawings and Documents

The CONSULTANT shall furnish the COMMISSION with the following completed sheets and documents, as applicable, for the construction project included in this contract, as follows:

- All mapping, sketches, cross sections and all other engineering documents necessary to secure permits from the administrator of the FEMA Flood Insurance Program (if required) and Section 401 and 404 permits as administered by the United States Army Corps of Engineers (if required).

- Bridge Deliverables

  - One (1) legible copy of the final design computations, coordinate geometry data and quantity computations in electronic format (pdf).

  - 100% complete unsigned and unsealed final bridge design plans in pdf format.

  - Final bridge design plans.  Electronic plans in MicroStation format and signed and sealed plans in pdf format will be submitted electronically.

  - Electronic plans (in MicroStation format) will be submitted after construction and include all bridge construction changes made to the plans during construction of the project.

  - Written job special provisions – bridge, as necessary to supplement the latest edition of the Missouri Standard Specifications for Highway Construction.

- Roadway Deliverables

  - One (1) electronic copy of the Conceptual Study Report for each project, including exhibits and cost estimate for review and approval. After any corrections, one (1) updated copy of the Conceptual Report, exhibits and cost estimate.

  - One (1) electronic copy of the preliminary roadway plans for each project, including any signing plans, temporary traffic control plans and preliminary cost estimate for review and approval. The COMMISSION shall prepare the sign details.  After any corrections, one (1) updated set of preliminary plans.

  - One (1) electronic copy of the Right of Way plans for each applicable project, including cross sections and driveway profiles and updated cost estimate for review and approval.  After any corrections, one (1) updated set with a sealed title sheet.

  - One (1) electronic copy of drainage and engineering calculations and analyses for each project.

  - One (1) electronic copy of the traffic control plans for review and comments for each project.  After any corrections, one (1) updated set will be required.

  - One (1) electronic draft copy of the job special provisions - roadway for review for each project (combined in one document if projects are let in combination).  After corrections, one (1) final sealed copy of the job special provisions shall be furnished. The PS&E submittal will be in word format.

  - One (1) electronic copy of the unsigned and unsealed roadway plans for initial review and comments for each project.  After corrections, one (1) fully checked, signed and sealed, original drawings of the final design plans shall be furnished.

  - One (1) electronic copy of the estimate of construction costs for the PS&E submittal for each project.  The estimate shall be prepared using the latest version of the Commission’s cost estimating software.

  - One (1) electronic copy of the workday study showing the estimated number of workdays required to construct each project.

  - One (1) electronic copy of the Transportation Management Plan for each project.

  - One (1) electronic copy of the D15 Equipment and Materials List (if applicable) for each project.

  - One (1) electronic copy of the Workzone Impact Analysis Spreadsheet for each project.

  - One (1) electronic copy of approved Design Exceptions (if applicable) for each project.

  - Floodplain Development Permit (if applicable).

# Standards

The CONSULTANT shall use the latest version of the following publications to determine the design criteria and procedures which will be followed for development of the project.

- Federal Emergency Management Administration Flood Insurance Guidelines and Specifications.

- MoDOT Engineering Policy Guide (EPG)

- Manual on Uniform Traffic Control Devices (MUTCD)

- A Policy on Geometric Design of Highways and Streets

- AASHTO LRFD Bridge Design Specifications

- Missouri Standard Specifications for Highway Construction

- Missouri Standard Plans

- MoDOT Bridge Manual

- Missouri Department of Transportation for Computer Deliverable Contract Plans

- Any other publications which the ENGINEER directs the CONSULTANT to use.