|  | Bridge No. |  |
| --- | --- | --- |
| Rev 08/18 | Job No. | J1S3216 |
|  | Replaces Bridge No. | T0195 |

# Missouri Department of Transportation

## Bridge Survey Report

| Bridge over |  | Walnut Fork |  |  |  |  |  |  |  |  |  |  |  | Route |  |  | E |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| County | Gentry |  |  | Sec. | 9 | Twp. | 62N |  | Rg. |  | 32W | ; | 2.45 |  | miles **SE of |  |  |  | Stanberry |
| *On road from |  |  | Stanberry 169 South – E East |  |  |  |  | to |  | Darlington |  |  |  |  |  | at Sta. |  | 58+78.86 |  |
|  |  |  | West or North of site |  |  |  |  |  |  | East or South of site |  |  |  |  |  |  |  |  |  |
| *Give adjacent towns each way, not terminal points of route. ** Delete all but one of N-E-S-W or circle appropriate direction. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surveyed by |  | Wilson & Company, Inc., Engineers & Architects |  |  |  |  |  |  |  |  |  |  |  | Date |  |  | 02/19/2021 |  |  |

| ### EXISTING MoDOT BRIDGE AT OR NEAR PROPOSED SITE (Data provided shall not come from existing bridge plans) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beginning Station |  | 58+24.00 |  |  |  | (ft) Ending Station |  |  |  | 59+45.38 |  |  |  |  | (ft) |
| Beginning Deck Elevation |  |  |  | 875.5’ |  | (ft) Ending Deck Elevation |  |  |  |  |  | 875.6’ |  |  | (ft) |
| Top of Sound Concrete Curb or Wing near Beginning Station |  |  |  |  |  |  |  | Top of Sound Concrete Curb or Wing near Ending Station |  |  |  |  |  |  |  |
|  | Elevation |  | 876.4’ |  | (ft) |  |  |  | Elevation |  | 876.3’ |  | (ft) |  |  |
|  | Station |  | 58+24.00 |  | (ft) |  |  |  | Station |  | 59+45.42 |  | (ft) |  |  |
|  | Offset |  | -9.79 |  | (ft) |  |  |  | Offset |  | -10.11 |  | (ft) |  |  |
| Does drift collect on structure? |  |  |  |  | N |  | Does the bridge back up water during flood? |  |  |  |  |  |  | N |  |
| Is the bridge overtopped during flood? |  |  |  |  | N |  | Frequency |  |  |  |  |  |  |  |  |
| Is the roadway overtopped during flood? |  |  |  |  | Y |  | Frequency |  | 5 years |  |  |  |  |  |  |

| #### HIGH WATER ELEVATIONS AT PROPOSED BRIDGE SITE If high water elevations are not available at proposed bridge site, give elevations where found and note location. |  |  |
| --- | --- | --- |
|  | #### Extreme High Water (Give date of occurrence) | #### Ordinary High Water Mark (See EPG 127.4.1.1) |
| #### Elevations | #### 2’ over road E East of Bridge | #### 853.53’ |
| #### Date(s) | #### 2019 | #### N/A |
| #### Location | #### Low point in E to the East | #### NORTH OF BRIDGE |
| #### Source of information | #### Tom Mcogdill | #### FIELD SURVEY |
| #### Head (or backwater from ) |  | #### N/A |
| #### Frequency (give dates) | #### 5 to 6 years | #### N/A |
| *** Character of drift | #### Light | #### N/A |

***Light – passes 12 ft opening;  Medium – passes 24 ft opening;  Heavy – requires over 24 ft opening

### IMPROVEMENTS WITHIN SURVEY AREA OF PROPOSED BRIDGE (WITHIN 1 FOOT ABOVE EXTREME HIGH WATER ELEVATION)

### Note the location and type of any improvements in the vicinity of the proposed bridge, including residences, businesses, other buildings, crop fields, etc.

### Homes East & West of bridge. Cultivated Fields in all quadrants at bridge.

| ### OTHER BRIDGES ACROSS SAME STREAM |  |  |
| --- | --- | --- |
| Information required for bridges as indicated on the Bridge Survey Location Request. Sketches of structure not required. See the Bridge Survey Location Request for additional data needed. |  |  |
|  | No. 1 | No. 2 |
| Distance along thalweg from proposed structure, upstream or down (ft) |  |  |
| Railroad, highway or pedestrian bridge. |  |  |
| Extreme High Water Elevation at structure |  |  |
| Does the bridge back up water during floods? |  |  |
| Additional Remarks: No other bridges |  |  |

| DATA FOR PROPOSED BRIDGE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Are the banks caving/sloughing at the site? |  | sloughing |  |  |  |  |  |
| Does the stream appear to be cutting or filling? |  | cutting |  |  |  |  |  |
| Elevation of extreme low water |  | (ft) During what months is stream dry? |  |  |  |  | none |
| Type of surface material of streambed (gravel, sand, silt, etc.) |  |  |  | dirt |  |  |  |
| Location of dam(s) having a definite spillway within 1 mile of the bridge site? |  |  |  |  | NA |  |  |
| If crossing is over drainage ditch, provide the corporate name of drainage district: |  |  |  |  |  | NA |  |

| Roadway Design Frequency and Required Permits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Roadway Design Frequency: | -Year (See EPG 748.2.2) |  |  |  |  |  |  |
| Corps of Engineers 404 Permit: | ☐ | Yes |  | ☐ | No |  |  |
| State Department of Natural Resources 401 Permit: | ☐ | Yes |  | ☐ | No |  |  |
| Environmental Protection Agency NPDES Permit: | ☐ | Yes |  | ☐ | No |  |  |

PHOTOGRAPHS OF SITE CONDITIONS

For grade crossings and retaining walls provide photographs documenting site characteristics as deemed necessary.

For stream crossings provide photographs documenting the site characteristics. Photos should be taken in an overlapping manner to provide a 360º panoramic view at or near the proposed stream crossing. Photos should also be taken to show the channel, banks and streambed both upstream and downstream of the proposed bridge, as well as the waterway through the existing bridge. If the existing roadway is overtopped at extreme high water, provide photographs showing the roadway on either side of the existing bridge. If the land use or stream characteristics are significantly different at upstream or downstream valley profiles, provide additional photographs to document these conditions. Additional photographs may also be necessary to provide information on other site-specific conditions. It is especially important to show any nearby improvements that may be affected by flooding or changes in stream velocity. Photos of other bridges near the proposed structure should also be included. These photos should show the bridge profile including details of the superstructure and substructure type. These photos should also show any bank or channel improvements or issues in the area.

Brief Description of Photographs (directions and locations):

Photos 1-7 panoramic view East to West looking Northerly. Photos 8-13 panoramic view West to East looking Southerly. Photo 14 Bridge deck looking North. Photo 15 Bridge deck looking South. Photo 16 West bank looking NE to East bank. Photo 17 East bank looking SW to West bank. Photo 18 East bank looking South. Photo 19 East bank looking South. Photo 20 looking North Photo 21 looking South Photo 22 looking West Photo 23 looking East 14 23 22 16 21 20 15 19 17-18 16 8-13 1-7

PHOTO 1

PHOTO 2

PHOTO 3

PHOTO 4

PHOTO 5

PHOTO - 6

PHOTO - 7

PHOTO - 8

PHOTO 9

PHOTO 10

PHOTO 11

PHOTO 12

PHOTO 13

PHOTO 14

PHOTO 15

PHOTO 16

PHOTO 17

PHOTO 18

PHOTO 19

PHOTO 20

PHOTO 21

PHOTO 22

PHOTO 23

CONDUIT UNDER BRIDGE, LOOKING EAST AT EAST ABUTMENT.

LOOKING WEST AT BRIDGE

LOOKING EAST, AT NE WINGWALL

GENERAL INSTRUCTIONS FOR BRIDGE SURVEYS

In order to provide the best possible structure design, it is important that this report be completed as fully and accurately as possible. Consultation with bridge office to resolve questions or issues that require considerable judgment is encouraged. The purpose of a bridge survey is to provide data needed to establish three important points: the general dimensions of the structure (length, height, skew, and arrangement of spans); the type, size and depth of foundation; and the cost of construction. For stream crossings these three points are very intimately related to the required waterway. A restricted waterway means serious scour, and footings must extend deep or be very substantially founded. Detailed instructions on completing the Bridge Survey Report and associated plan and profile sheets are contained in EPG 747 Bridge Reports and Layouts of the Engineering Policy Guide.