# Bridge Memorandum                                      Sheet 1 of 2

Job No: J7P3107F						 Bridge No: A9115

Route:   B (Minor) over Marais des Cygnes River   		 County: Bates

Final Layout: (100’-115’-60’) Prestressed Concrete NU43 Girder Spans

Roadway Width: 24 ft. (symmetrical) plus 16” Type D Barriers

Alignment:  Tangent

Skew: 25° R.A.  (℄ Channel Sta. = 221+02.98 +/- ℄ Rte. B)

Loading: HL-93

Profile Grade:  V.P.I. Sta. 218+11.74, Elev. 751.65, +3.69% Bk., 0.00% Ahd. L=185.58’,

V.P.I. Sta. xxx+xx.xx, Elev. xxx.xx, 0.00% Bk., +1.00% Ahd. L=xxx.xx’,

V.P.I. Sta. xxx+xx.xx, Elev. xxx.xx, +1.00% Bk., +1.67% Ahd. L=xxx.xx’&

V.P.I. Sta. 223+66.53, Elev. 757.13, +1.67% Bk., 0.00% Ahd. L=258.79’

Beg. Station: 219+35.94 at Fill Face of End Bt. 1 ℄ Rte. B

Fill Exception: Sta. 219+35.94 to Sta. 222+14.80 +/-

Traffic Handling: Structure to be closed to traffic during construction.  See roadway plans for traffic control.

Existing Bridge: T0280 to be removed per standard specs, estimated cost $115,920 (Bridge Item, included in estimate).

General Notes:

•  Stationing, profile grade, and centerline structure are located along centerline Route B.

# •  End fills shall be 2.5:1 normal to end bents. (Roadway Item)

# •  Provide 2’ minimum thick Type 2 rock blanket with Permanent Erosion Control

# Geotextile extending to toe of slope on each side of channel (match Bridge T0280 side

# slope elevation minimum elev. 751). (Roadway Item)

•  Channel to be cleaned out. (Roadway Item)

•  Provide berms on bridge ends 4’ min. below bottom of girder.

•  AADT (2022) = 531; AADT (2042) = 587, AADT Truck = 16.3%.

•  Provide right-of-way as required for construction. (Roadway Item)

•  Relocate all utilities as required for construction. (Roadway Item)

•  Use integral end bents with straight turned back wings 15’ from fill face to wing end.

# •  Provide Bridge Approach Slabs (Minor Road). (Bridge Item).

•  No conduit, lighting, utility supports, or sidewalks are to be included in the final plans for

this bridge.

•  Asbestos and lead inspections have been performed on this structure (Bridge

T0280). Results indicate that lead based paint is present. The Bridge Division will include

the inspection report in the electronic deliverables folder when submitting contract

documents to the Design Division for the Letting (Bridge Item).

•  An NFIP Flood Insurance Rate Map for Bates County, MO (FIRM Panel 29013C0450C,

Effective date May 03, 2010) shows this site in a “Zone A” flood hazard area subject

to 100-year flooding. A floodway has not been identified.

Bridge Division will obtain Floodplain Development Permit.

•  3-girder lines assumed.

•  Note similar bridges A8530 & A8724.

•  Refer to Bridge A8494 records which is an overflow structure west on Route B and

the hydraulics are related. Refer also to bridge memo for A9113.

•  Provide for web walls at intermediate bents based on history of drift.

# Bridge Memorandum - Continued                Sheet 2 of 2

### Hydrologic Data

### Drainage Area = 3690.0 mi2

Design Flood Frequency =50 years Design Flood Discharge =132,500 cfs Design Flood (D.F.) Elevation =745.6

Base Flood (100-year)

Base Flood Elevation =746.2 Base Flood Discharge =157,000 cfs Estimated Backwater =0.2 ft Average Velocity thru Opening =3.8 ft/s

### Freeboard (50-year)

Freeboard =2.0 ft

### Roadway Overtopping

Overtopping Flood Discharge =45,000 cfs Overtopping Flood Frequency =3 years Overtopping Flood Elev. =744.0

*FY22 Estimated Construction Cost: $1,120,000.

Programmed Bridge STIP Amount (Total Project): $3,150,000

Programmed Total Inc. Roadway STIP Amount: $3,624,000

(Working day Est. 75 and Calendar Day 105)

* Does not include STIP inflation factor from planning (3% compounded annually)

District contact is Shannon Kellner, TPM (660) 885-5665.

Bridge contact is Joe Alderson, SPM (573) 522-8722.

Bridge Wayne V. Elliott, III             	 Date April 15, 2021

Prepared by Wayne V. Elliott, III Bridge Location and Layout Designer

Bridge __________________________ Date _______________

Structural Project Manager

District __________________________Date _______________

Transportation Project Manager

District __________________________Date _______________

District Bridge Engineer