MISSOURI DEPARTMENT OF TRANSPORTATION

STEEL FRAME, STEEL FLOOR PLATFORM BODY SPECIFICATIONS

FOR SINGLE REAR WHEEL TRUCK

Overall width not to exceed 79".

Overall length as specified on bid form.

Body frame to be all steel welded into a single unit. Bulkhead may bolt onto body frame.

Long sills shall be 4", 6.7# structural steel and gusseted front, rear and at every other cross sill.

Cross sills shall be 12 gauge high strength {50,000psi} roll formed steel spaced on minimum 18" centers for maximum strength and gusseted throughout to long sills and welded to side rails.

Other variations of long sills and cross sills will be accepted if strength is equal to or greater than those specified.

Floor, rear and sides (rub rails) shall be a minimum of 12 gauge high strength, {50,000psi} roll formed steel tread plate with reinforced internal stake pockets sized to accept 2”x4” lumber or steel stake racks.

Floor shall be non-skid steel tread plate (minimum 1/8”).

Rear skirt, one piece design, recessed for light protection.

Body shall include all required lights and reflectors according to Federal Motor Vehicle Safety Standards No. 108.  Recessed, rubber grommeted, weather tight and high impact reflectorized LED lights where applicable.

Backup alarms are to be provided. Rear flaps are to be provided (preferred 1/2” x 20” x 30”).

The entire bed shall have either complete immersion in electro deposition prime(E-coat) or powder coat paint system. Undercoating on underside of platform. Durable high gloss black finish.

Descriptive literature and complete technical data should be furnished with your bid.

Bids will be considered on any standard unit providing that any minor deviations from these specifications are clearly noted on bid.

The Missouri Highway and Transportation Commission reserves the right to waive technicalities and to reject any or all bids and no bid is final until formally accepted by the Commission.